# SYSTEM REQUIREMENT DOCUMENT

By

Tanner Whitney
Zachary McKissor
Charles Allen
Ryan Coder
Owen Glascoe
Isaiah Gale

Submitted to Dr. Siwei Fan and Dr. Matthew Haslam
of Embry-Riddle Aeronautical University
in Partial Fulfillment of the Course Requirements for AE 445 and COM 430

January 30, 2026

# ABSTRACT

The Proximity & Rendezvous Integrated Simulation Model (PRISM) testbed is designed to provide an on-campus platform for conducting controlled experiments in satellite translational and rotational control, supporting the study of Rendezvous Proximity Operations (RPO) in a laboratory environment. PRISM integrates a MockSat vehicle, an Air Table, and a Ground-Truth Sensing System (GTSS) to provide a near-frictionless 2D environment for validating Guidance, Navigation, and Control (GNC) algorithms. The System Requirement Document (SRD) outlines the finalized requirements for PRISM, including the main objective, Testbed-level requirements, and system-level requirements. Validation of each requirement will be proven through either test, demonstration, or inspection. The SRD is written to support iterative development and reproducible experiments for use by future students and faculty, providing the contractual baseline that guides design decisions, trade studies, and acceptance testing for PRISM.

# TABLE OF CONTENTS

ABSTRACT	ii

TABLE OF CONTENTS	iii

LIST OF FIGURES	iv

LIST OF TABLES	v

LIST OF NOMENCLATURE	vi

LIST OF ABBREVIATIONS	ix

1.0	Purpose	11

2.0	Objectives	11

3.0	System Architecture	12

3.1	Physical Architecture	12

3.2	Functional & Logic Architecture	15

4.0	Testbed Requirements	15

5.0	System Requirements	17

5.1	Air Table	17

5.2	MockSat	18

5.3	Ground Truth Sensing System (GTSS)	20

REFERENCES	22

APPENDIX	23

Attributions Table	23

AI Attributions Table	23

Acknowledgements	23

Organizational Chart	24

Hours to Date	24

Itemized Budget	25

Requirements Verification Matrix	26

# LIST OF FIGURES

Figure 1: MockSat Side Cut View	12

Figure 2: MockSat Side Cut View	13

Figure 3: Plenum CAD Model	13

Figure 4: GTSS Isometric Side Cut View	14

Figure 5: PRISM's Organizational Chart	24

# LIST OF TABLES

Table 1: Attributions Table	23

Table 2: AI Attributions Table	23

Table 3: PRISM Hours to Date	24

Table 4: Itemized Budget	25

Table 5: Requirements Verification Matrix	26

# LIST OF NOMENCLATURE

| MockSat | A satellite mockup used to simulate in-space code and dynamics on a testbed on Earth. |
| --- | --- |
| Air Table | A flat platform with an air-bearing system that simulates a near-frictionless space environment for spacecraft testing. |
| Rendezvous Proximity Operations (RPO) | Techniques and procedures that allow one spacecraft to approach, maneuver around, and potentially dock with another spacecraft. |
| Flight Computer | The onboard Raspberry Pi 5 computer that executes commands, runs control algorithms, manages subsystem operations, and logs data. |
| Plenum | The sealed, pressurized internal chamber of the Air Table that distributes compressed air beneath the porous aluminum surface. |
| Air Gap | The vertical distance between the MockSat base and the Air Table surface, maintained by pressurized airflow to create near-frictionless motion. |
| Baffle | A perforated plate inside the Air Table plenum that reduces air concentration and ensures uniform pressure distribution across the table surface. |
| APRIL Tag | An open-source fiducial marker system that uses QR-code-style patterns to enable precise position and attitude tracking through computer vision. |
| Testbed | The integrated PRISM platform consisting of the Air Table, MockSat, and Ground-Truth Sensing System used for simulating orbital operations. |
| Maneuvers | Controlled movements executed by the MockSat using its propulsion system to achieve desired changes in position or orientation. |
| Trajectory | The path the MockSat follows through the X-Y plane across the testbed. |
| Attitude Control | The orientation control of the MockSat using differential thrust from propellers to achieve rotation about the Z-axis. |
| Translational Control | The position control of the MockSat using coordinated thrust to achieve motion along the X and Y axes. |
| Coast Acceleration | The parasitic deceleration experienced by the MockSat due to residual drag forces from air friction and viscous effects. |
| Impulse ΔV | The incremental change in velocity produced by a short propulsion burst, representing the smallest controllable velocity change. |
| Total ΔV Budget | The cumulative velocity change capability of the MockSat based on available thrust, mass, and operational duration. |
| Yaw | Rotation of the MockSat about its vertical (Z) axis. |
| Degrees of Freedom (DOF) | The number of independent motion parameters; PRISM provides three DOF: X-translation, Y-translation, and Z-rotation. |
| Thrust Repeatability | The consistency of thrust output for a given command, measured as the standard deviation relative to mean thrust. |
| Run Time | The maximum continuous duration the MockSat can operate its propulsion system at full throttle. |
| Lift Capacity | The maximum payload mass the Air Table can support while maintaining proper air gap and stable levitation. |
| Surface Deflection | The maximum vertical displacement of the Air Table surface under load. |
| Synchronization | The process of aligning timestamps between the MockSat flight computer and GTSS to enable direct comparison of telemetry data. |
| State Estimation | The process of determining the MockSat's position, velocity, and orientation from sensor measurements. |
| Control Algorithm | Software logic, such as PID control, that calculates required thrust commands to achieve desired motion. |
| Test Campaign | The structured sequence of experiments designed to validate MockSat performance, stability, and control responsiveness. |
| Sensor Calibration | The process of configuring the IMU and other sensors to establish accurate baseline measurements before testing. |
| Guidance, Navigation, and Control (GNC) | The integrated set of techniques for determining vehicle state, planning trajectories, and executing control commands. |
| Parasitic Drag | Resistive forces opposing motion, including air friction and viscous drag in the thin air gap. |
| Command Station | The operator interface and workstation used to configure the MockSat, monitor telemetry, and analyze test results. |
| Pitch and Propeller Characteristics | Physical parameters of the propellers that determine thrust output as a function of motor voltage and rotation speed. |
| Electronic Speed Controller (ESC) | Hardware that regulates motor current and translates PWM signals into motor control commands. |
| Pulse-Width Modulation (PWM) | A signal modulation technique used to control motor speed by varying the duty cycle of electrical pulses. |
| Inertial Measurement Unit (IMU) | A sensor package that measures linear acceleration and angular rotation rates to support state estimation. |
| Stagnation Pressure | The maximum pressure achieved when moving air is brought to rest inside the plenum. |
| Gauge Pressure | Pressure measured relative to atmospheric pressure rather than absolute vacuum. |
| Moment of Inertia | A measure of the MockSat's resistance to angular acceleration about its vertical axis. |
| Angular Acceleration | The rate of change of rotational velocity, produced by net torque from differential thrust. |
| Torque | The rotational force generated by thrust offset from the MockSat's center of mass. |
| Differential Thrust | The propulsion technique of increasing thrust on one side while decreasing it on the opposite side to produce rotation without net translation. |
| PID Control | A control algorithm using proportional, integral, and derivative feedback terms to minimize error between commanded and actual states. |
| Simulink Model | A simulation environment used to test control algorithms and predict MockSat dynamics before hardware testing. |
| Robot Operating System 2 (ROS 2) | Middleware framework that facilitates communication between sensors, processors, and actuators, and enables organized data logging. |
| Bag Files | ROS 2 data files that record timestamped sensor and telemetry data for later analysis and replay. |
| Footprint | The circular base area of the MockSat that contacts the air cushion on the Air Table surface. |
| Bumpers | Protective wooden barriers around the Air Table perimeter that prevent the MockSat from leaving the test surface. |

# LIST OF ABBREVIATIONS

| PRISM | Proximity & Rendezvous Integrated Simulation Model |
| --- | --- |
| RPO | Rendezvous Proximity Operations |
| GTSS | Ground-Truth Sensing System |
| GNC | Guidance, Navigation, and Control |
| DOF | Degrees of Freedom |
| IMU | Inertial Measurement Unit |
| ESC | Electronic Speed Controller |
| PWM | Pulse-Width Modulation |
| PID | Proportional-Integral-Derivative |
| ROS 2 | Robot Operating System 2 |
| PDB | Power Distribution Board |
| CAD | Computer-Aided Design |
| NASA | National Aeronautics and Space Administration |
| APRIL | A Pose Recognition and Identification Library |
| QR | Quick Response (code) |
| FOV | Field of View |
| UI | User Interface |
| LiHV | Lithium High Voltage |
| KV | Motor Velocity Constant (RPM per Volt) |
| PCA9685 | 16-Channel 12-bit PWM Driver Integrated Circuit |
| TBR | To Be Resolved |
| BNO055 | Bosch 9-axis Inertial Measurement Unit Model Number |
| RPM | Revolutions Per Minute |
| CFM | Cubic Feet per Minute |
| Hz | Hertz (frequency unit) |
| kg | Kilogram (mass unit) |
| m | Meter (length unit) |
| s | Second (time unit) |
| N | Newton (force unit) |
| Pa | Pascal (pressure unit) |
| cm | Centimeter (length unit) |
| mm | Millimeter (length unit) |
| V | Volt (electrical potential unit) |
| W | Watt (power unit) |
| A | Ampere (electrical current unit) |
| mAh | Milliampere-hour (battery capacity unit) |
| ° | Degree (angular unit) |
| °/s | Degrees per second (angular velocity unit) |
| °/s² | Degrees per second squared (angular acceleration unit) |
| m/s | Meters per second (velocity unit) |
| m/s² | Meters per second squared (acceleration unit) |
| cm/s | Centimeters per second (velocity unit) |
| cm/s² | Centimeters per second squared (acceleration unit) |
| kg·m² | Kilogram meter squared (moment of inertia unit) |
| N·m | Newton meter (torque unit) |
| m² | Square meter (area unit) |
| m³/s | Cubic meters per second (volumetric flow rate unit) |
| Pa·s | Pascal second (dynamic viscosity unit) |
| 1σ | One standard deviation (statistical measure) |

# Purpose

The Proximity & Rendezvous Integrated Simulation Model (PRISM) establishes a foundation for exploring Rendezvous Proximity Operations (RPO) theories and techniques that allow one spacecraft to approach, maneuver around, and potentially dock with another spacecraft. RPO serves as a cornerstone of modern and future space missions because it enables on-orbit servicing, refueling, debris removal, satellite inspection, and assembly of large orbital structures.

RPO technologies advanced significantly during NASA’s Gemini missions in the 1960s. Engineers developed the basic methods for co-elliptic rendezvous from above and below and demonstrated real-time maneuvering using ground-based and onboard sensors [2]. NASA also refined spacecraft docking and station-keeping procedures to protect crew safety and maintain accurate trajectories [2]. These developments enabled the Apollo 9–15 missions and formed the basis of modern RPO [2]. Although these early programs made major progress, opportunities for improved precision, autonomy, and system reliability remain.

Modern space missions rely on RPO for debris capture and removal, in-orbit assembly, repair and refueling, formation flying, and spacecraft docking. Current research focuses on autonomous RPO technologies to address navigation uncertainties, real-time collision avoidance, and integration with low-size, low-weight, and low-power hardware [3]. These autonomous methods aim to improve precision, reliability, and scalability for future mission demands.

Government and commercial organizations continue to invest heavily in these technologies. Northrop Grumman, for example, partnered with the U.S. Space Force to develop a more efficient satellite refueling method through the Elixir program, which aims to extend the operational life of satellites in geosynchronous orbits [4]. As these investments grow, students must gain hands-on experience with RPO-related systems and techniques.

Embry-Riddle Aeronautical University does not have an on-campus testbed for 2D translation and 1D rotation satellite maneuvers. Students currently rely on external facilities, such as Honeywell’s satellite attitude controller testbed, to validate designs. These trips depend on scheduling availability, travel time, and limited test duration, which restrict participation and prevent iterative improvements within a semester. This lack of accessible infrastructure slows the development of advanced satellite control skills and reduces hands-on learning opportunities. PRISM aims to resolve this gap by providing an on-campus testbed that enables controlled experiments, system-level performance measurement, and efficient design iteration.

# 2.0	Objectives

PRISM is driven by one objective that defines the scope of the project and from which all testbed and system level requirements are derived.

OB1.0 (Testbed Surface) The Testbed shall provide a near-frictionless surface that enables 2DOF translational motion and 1DOF rotational motion.

OB1.0 specifies how the Testbed shall emulate a simplified space environment.

OB2.0 (Testbed Vehicle) The Testbed shall provide a vehicle representative of a spacecraft capable of executing predefined control algorithms.

OB2.0 specifies how the Testbed will execute predefined user algorithms.

# 3.0	System Architecture

The PRISM testbed consists of three independent systems, each containing its own subsystems, functions, and interfaces. This architecture isolates responsibilities, reduces integration complexity, and establishes clear boundaries for testing and verification.

## 3.1	Physical Architecture

The MockSat system displayed in Figures 1 and 2 contains propulsion, power, avionics, telemetry, and user interface subsystems.

Figure 1: MockSat Side Cut View

Figure 2: MockSat Side Cut View

The propulsion subsystem consists of eight propellers, eight motors, eight electronic speed controllers (ESCs), and one pulse-width modulation (PWM) board. These components mount to the vehicle structure and generate the forces and torques required for planar translation and yaw rotation. The power subsystem includes three lithium polymer (LiPo) batteries, one voltage regulator circuit, and two power distribution boards (PDBs), which supply power to all onboard systems. The avionics subsystem consists of a single Raspberry Pi 4 microcontroller that serves as the vehicle’s central processor for control execution, data logging, and command handling. The telemetry subsystem includes a single inertial measurement unit (IMU) used for state estimation. The user interface subsystem consists of an on-off switch, a button array, and a display board mounted to the vehicle to allow operators to configure modes and initiate tests.

The Air Table Plenum in Figure 2 is a 1m × 1m engineered test surface constructed as a reinforced box structure that functions similarly to a highly controlled air-hockey table. The frame is built from 2” × 4” wooden planks, forming a rigid perimeter that supports a sealed plywood interior designed to act as a pressurized plenum.

Figure 3: Plenum CAD Model

The plywood is fully sealed using caulk along all internal seams and coated with paint to create an airtight chamber. Pressurized air from an external compressor is routed through regulators, valves, and air lines into this internal plenum, where it distributes evenly beneath a porous aluminum sheet that forms the table surface. The aluminum sheet is sealed to the frame using epoxy and mechanically fastened with bolts that also secure protective wooden bumpers around the perimeter. These bumpers serve two purposes: they provide a secondary mechanical seal for the top plate and prevent the MockSat from sliding off the table in the event of a fault or instability. When pressurized, the porous aluminum produces a uniform cushion of air through its micro-perforations, enabling the MockSat to translate in two axes and rotate freely with minimal friction while remaining confined within the boundaries of the table.

The GTSS in Figure 3 consists of a camera, mounting hardware, synchronization electronics, a processing workstation, and auxiliary electrical equipment, all working together to provide accurate external reference data for vehicle position and attitude.

Figure 4: GTSS Isometric Side Cut View

The camera is mounted above the Air Table on a rigid structure to maintain a fixed and calibrated view of the test area. A synchronization module aligns the GTSS timestamps with the internal clock of the MockSat so that externally measured states can be directly compared to onboard estimates. During operation, the camera continuously captures images of QR code style fiducial markers attached to the upper surface of the MockSat, and computer vision algorithms use these markers to compute precise translational and rotational states. A dedicated workstation processes the images, performs pose estimation, filters the resulting data, and stores the high-fidelity ground truth trajectory for analysis, validation, and controller performance assessment.

## 3.2	Functional & Logic Architecture

In the logic architecture, the MockSat operates as an autonomous-control node, the Air Table acts as the environmental domain, and the GTSS acts as an external measurement node. Functionally, each system performs a distinct role in the Testbed. The MockSat executes control algorithms, receives operator inputs, and produces thrust commands to maneuver across the Air Table. The vehicle’s onboard logic begins when the operator configures the system using the user interface. The Raspberry Pi receives these inputs, loads the corresponding control mode, and begins data logging. During motion, the IMU provides acceleration and angular rate data that the Raspberry Pi uses for state estimation and control feedback. The processor sends PWM signals to the ESCs through the PWM board, causing the motors and propellers to generate the commanded forces and torques. The power subsystem provides isolated power to propulsion, avionics, and user interface components throughout operations.

The Air Table’s functional role is to create a low-friction environment that emulates planar microgravity dynamics. Pressurized air flows from the compressor to the plenum, then through the nozzle array to lift the MockSat and minimize resistive forces.

The GTSS performs high-fidelity motion capture and provides an external reference for validating onboard telemetry. Cameras record the MockSat’s motion, and the GTSS computer processes the imagery to determine precise position and orientation over time. Synchronization hardware aligns GTSS timestamps with those recorded by the MockSat’s flight computer, enabling direct comparison of trajectories, state estimates, and control performance.

# 4.0	Testbed Requirements

To achieve the function of the Air Table, MockSat, and Ground Truth Sensing System, the Testbed-level requirements are defined based on the objective. The requirements will determine the accomplishment of PRISM. The requirements are outlined below and will be examined further at the system level.

R1.1 (Operational Environment) The Testbed shall simulate two-dimensional, frictionless motion on a planar surface.

Requirement 1.1 is derived from OB1.0 (Testbed Surface). R1.1 specifies the test environment of the testbed. R1.1 will be verified through demonstration. This demonstration will consist of placing the MockSat on the Air Table’s surface and observing that the MockSat levitates over the Air Table’s surface when the Air Table is turned on.

R1.2 (Translational Maneuvers) The Testbed shall support the execution of pre-defined and user-programmed, point to point, translational maneuvers with no more than 10 mm of steady state error.

Requirement 1.2 is derived from OB2.0 (Testbed Vehicle). R1.2 specifies the testbed’s capability to perform preprogrammed, translational maneuvers developed by users. R 1.2 will be verified through a test. The test will consist of commanding the MockSat to perform a purely translational maneuver to a specified position on the Air Table. Then, a measurement will be taken to verify the steady state error is less than 10 mm.

R1.3 (Rotational Maneuvers) The Testbed shall support the execution of pre-defined and user-programmed, rotational maneuvers with no more than 2° of steady state error.

Requirement 1.3 is derived from OB1.0 (Testbed Vehicle). R1.3 specifies the testbed’s capability to perform preprogrammed, rotational maneuvers developed by users. R1.3 will be verified through a test. The test will consist of commanding the MockSat to perform a purely rotational maneuver to a specified attitude on the Air Table. Then, a measurement will be taken to verify the steady state error is less than 2°.

R1.4 (ΔV Budget) The MockSat shall supply itself with a total capable change in linear velocity of at least 10 m/s.

Requirement 1.4 is derived from OB2.0 (Testbed Vehicle). R1.4 specifies the minimum total change in linear velocity the MockSat needs to supply for each test. R1.4 will be verified by a demonstration. The demonstration will consist of commanding the MockSat to perform multiple maneuvers requiring a total change in linear velocity of 10 m/s. This requirement will be verified if the MockSat completes all maneuvers of the test.

R1.5 (Dynamic Environment) The Testbed shall produce no more than 0.098 m/s2 of coast deceleration.

Requirement 1.5 is derived from OB1.0 (Testbed Surface). R1.5 specifies the need for the MockSat to experience a near frictionless environment and to do this the MockSat must have a coast deceleration of no more than 0.098 m/s2. Coast acceleration, otherwise known as parasitic acceleration, determines how frictionless the simulated environment is by examining the drag of the MockSat and determining whether it is negligible. Per Huang [6], for air table simulation test beds, parasitic drag will not exceed 1% of the normal force of the MockSat. 1% of 49.05 N is 0.4905 N and deceleration equals force divided by a maximum MockSat mass of 5kg which equals 0.098 m/s2. This number will be verified through test by recording the speed of the MockSat moving unpowered across the table and finding the coast deceleration it experiences by finding the slope of the distance and time relation.

R1.6 (Repeatability) The Testbed shall maintain consistency in operating performance between tests defined as a 5% deviation in travel-time from point to point.

R1.6 is derived from OB2.0 (Testbed Vehicle). R1.6 specifies the Testbed’s need complete multiple tests with the same timeframe across replicated conditions. This will be verified through test. The MockSat will be commanded to run the same translational and rotational maneuvers multiple times. This requirement will be verified if the variability of the time it takes to complete each replicated test is below 5%.

# 5.0	System Requirements

The system level requirements will justify the function of Air Table, MockSat, and Ground Truth Sensing System. The system level requirements will determine the success of PRISM systems. Each system has its own section as follows.

## 5.1	Air Table

To ensure the Air Table system meets the testbed requirements, the subsystem levels are defined. These requirements establish performance, functionality, and verification criteria for the Air Table.

R2.1 (Freedom of Motion) The Air Table shall provide an operating area of at least 1 m2.

Requirement 2.1 is derived from R1.1 (Operational Environment). R2.1 specifies the need for the MockSat to have adequate maneuvering space to perform maneuvers. The air-table planform area of 1 m² was selected primarily in response to project budget constraints. A 1 m × 1 m sheet of 12 mm 6061 aluminum costs approximately half the price of a comparable 1.5 m × 1.5 m sheet, making larger sizes financially impractical. A 1 m by 1 m table area satisfies R2.1 while keeping the cost down. This will be verified through test by measuring the facing with a measuring tape to verify that it is 1 m in each direction and 12.7 mm thick. 
 
R2.2 (Payload) The Air Table shall support payloads up to 0.0154 kg/cm2.

Requirement 2.2 is derived from R1.1 (Operational Environment). R2.2 specifies the need for the Air Table to be able to put enough pressure on the MockSat to make it levitate. CubeSats of similar size to the MockSat are a maximum of 5 kg when made of PETG or PLA plastic. Since the MockSat will be made of PETG or PLA, the dimensions of our MockSat are standard for CubeSats, and our footprint is 8 inches in diameter, the team determined that the maximum weight per area of footprint our table can lift is 0.0154 kg/cm2. R2.2 will be verified through test. This requirement will be tested by creating a MockSat that has a weight per area footprint of 0.0154 kg/cm2 and verifying the system can lift it.

R2.3 (Air Gap) The Air Table shall maintain an air gap with the payload between 0.18 mm and 0.4 mm.

Requirement 2.3 is derived from R1.1 (Operational Environment). R2.3 specifies the need to maintain the frictionless environment. The air gap cannot exceed the range in the requirement without losing the near frictionless characteristics of the Air Table. This range was determined by examining similar systems and observing that the air gaps of those testbeds were always within this range for the size of table that the Air Table is. R2.3 will be verified by test. Verification will be performed by measuring the air gap using either a feeler gage being slipped between the MockSat and Air Table while operating or by having a laser point at the MockSat while the table is off and marking the location on the MockSat. Then turn the table on and mark the new location on the MockSat and measure the difference.

R2.4 (Surface Flatness) The Air Table facing shall maintain a surface flatness of ±0. 1 mm within an 8-inch diameter circle.

Requirement 2.4 is derived from R1.1 (Operational Environment). R2.4 specifies the need for the Air Table to simulate the near-frictionless environment. Surface flatness is critical, as any unevenness in the facing would introduce unintended forces and reduce the fidelity of the simulated environment. R2.4 will be verified through test. To test the surface flatness, the team will use lasers to examine 8-inch diameter sections on the facing to see if the laser is impeded.

## 5.2	MockSat

The MockSat must demonstrate controlled translation and rotation maneuvers on the Air Table. The individual subsystem requirements for the MockSat that were derived from the system requirements are listed below.

R3.1 (IMU Accuracy) The MockSat controller shall be capable of maintaining a commanded position and attitude within 10mm of steady state error.

R3.1 is derived from R1.2 (Translational Maneuvers) and R1.3 (Rotational Maneuvers). R3.1 specifies that the MockSat controller must ensure the position and attitude of the MockSat stays within 10 mm of the desired position and attitude of the user. This will be verified through test. The test will consist of commanding the MockSat to maneuver to a pre-determined position and attitude, measuring the difference to ensure a maximum of 10 mm error.

R3.2 (Motor Actuation) The propellers shall produce at least 1 N of thrust to translate and rotate the MockSat in the direction desired by the user.

R3.2 is derived from R1.4 (ΔV Budget). R3.2 specifies that the propulsion sub-system must be capable of correctly translating and rotating the MockSat on command. This will be verified through test. The MockSat will be commanded to both translate and rotate. Successful completion of the maneuver will prove the MockSat’s ability to provide sufficient thrust for testing.

R3.3 (Response Time) The MockSat will respond to any direction given by the controller within 100 milliseconds.

R3.3 is derived from R1.2 (Translational Maneuvers) and R1.3 (Rotational Maneuvers). R3.2 specifies the need for the MockSat to be capable of responding quickly to the IMU commands. This will be verified through test. PRISM will time how long it takes for the MockSat to complete a command after it has been sent from the IMU.

R3.4 (Rotational Motion) The MockSat shall provide itself with 1DOF rotational motion.

Requirement 3.4 is derived from R1.2 (Translational Maneuvers). R3.4 specifies that the MockSat’s propulsion subsystem is responsible for providing the MockSat with 1DOF rotational motion. R3.4 will be verified by demonstration. The demonstration will consist of commanding the MockSat to execute rotational maneuvers about its vertical axis to confirm motion consistent with commands.

R3.5 (Attitude Control) The MockSat shall maintain commanded attitude on the Air Table with no more than 2° steady state error.

Requirement 3.5 is derived from R1.2 (Translational Maneuvers). R3.5 specifies the accuracy to which the MockSat must control its own steady state attitude. R3.5 will be verified by a test. The test will consist of commanding the MockSat to maneuver to a specified attitude. Then, a measurement will be taken to verify the steady state error is less than 2°.

R3.6 (Linear Acceleration) The MockSat shall linearly accelerate to at least 5 m/s2 within 500 milliseconds.

Requirement 3.6 is derived from R1.2 (Translational Maneuvers). R3.6 specifies the minimum rate at which the MockSat must linearly accelerate to perform quick translational maneuvers. R3.6 will be verified by a test. The test will consist of commanding the MockSat to complete a purely translational maneuver at full throttle and measuring the MockSat’s acceleration. This requirement will be verified if the linear acceleration exceeds 5 m/s2.

R3.7 (Angular Acceleration) The MockSat shall angularly accelerate to at least 2°/s2 within 500 milliseconds.

Requirement 3.7 is derived from R1.3 (Rotational Maneuvers). R3.7 specifies the minimum rate at which the MockSat must angularly accelerate to perform quick rotational maneuvers. R3.7 will be verified by a test. The test will consist of commanding the MockSat to complete a purely rotational maneuver at full throttle and measuring the MockSat’s acceleration. This requirement will be verified if the angular acceleration exceeds 2°/s2.

R3.8 (Thrust Operation) The MockSat shall produce maximum thrust for at least 2.5 minutes.

Requirement 3.8 is derived from R1.2 (Translational Maneuvers) and R1.3 (Rotational Maneuvers). R3.8 specifies the minimum run time of the MockSat at continuous full throttle operation. R3.8 will be verified by a demonstration. The demonstration will consist of commanding the MockSat to fire at least two motors at full throttle for 2.5 minutes. This requirement will be verified if the MockSat completes the demonstration.

## 5.3	Ground Truth Sensing System (GTSS)

The GTSS must validate the real time position and attitude of the MockSat throughout the full duration of a test. The individual subsystem requirements for the GTSS system are explained below.

R4.1 (Data Logging) The GTSS shall provide time stamped position and attitude data of the MockSat throughout the entire duration of each test.

Requirement 4.1 is derived from R1.2 (Translational Maneuvers) and R1.3 (Rotational Maneuvers). R4.1 specifies the need to continually record data throughout each test. An open-source software, APRIL Tag, will provide the desired position and attitude data from the MockSat. APRIL Tags use QR-codes to act as coordinate frames for an object under test. The algorithm will transmit data every 0.5 seconds, allowing the potential for continuous tracking. R4.1 will be verified through test. The test will include placing April Tag QR-codes on the MockSat and using a camera with the April Tag’s software uploaded to verify that position and attitude data is being transmitted every 0.5 seconds.

R4.2 (Field of View) The GTSS shall be able to view the entire surface area of the Air Table.

Requirement 4.2 is derived from R1.1 (Operational Environment).  R4.2 specifies that the GTSS requires the capability to measure the MockSat position and attitude at any point on the surface of the table. R4.2 will be verified through demonstration. The demonstration will include taking a photo with the camera when the GTSS system is in place above the Air Table. Observing the entirety of the Air Table in the photo will verify a complete field of view.

R4.3 (Translational Accuracy) The GTSS shall provide recorded position accuracy within 5 mm.

Requirement 4.3 is derived from R1.2 (Translational Maneuvers) and R1.3 (Rotational Maneuvers). R4.3 specifies the need to have accurate position data to compare to the position of the MockSat. Without an accurate location, PRISM will not be able to correctly determine if the MockSat performs the maneuvers accurately. R4.3 will be verified through test. The test will involve the MockSat completing a simple translation maneuver while the GTSS system is collecting data. The data collected from the April Tags will be compared to the position data of the MockSat to determine if the tolerance is met.

R4.4 (Attitude Accuracy) The GTSS shall provide recorded attitude accuracy within 2°.

Requirement 4.4 is derived from R1.2 (Translational Maneuvers) and R1.3 (Rotational Maneuvers). R4.4 specifies the need to have accurate attitude data to compare to the attitude of the MockSat. Without an accurate truth, PRISM will not be able to correctly determine if the MockSat performs the maneuvers accurately. R4.4 will be verified through test. The test will involve the MockSat completing a simple rotation about the yaw axis maneuver while the GTSS system is collecting data. The data collected from the April Tag’s will be compared to the attitude data of the MockSat to determine if the GTSS is within tolerance.

# REFERENCES

[1] Huang, Z., Zhang, W., Chen, T., Wen, H., and Jin, D., “Characterizing an Air-Bearing Testbed for Simulating Spacecraft Dynamics and Control,” Aerospace, Vol. 9, No. 5, 2022, p. 246. https://doi.org/10.3390/aerospace9050246

[2] Goodman, J. L., “History of Space Shuttle Rendezvous and Proximity Operations,” Journal of Spacecraft and Rockets, Vol. 43, No. 5, 2006, pp. 944–959. https://doi.org/10.2514/1.19653

[3] Gialvaji, M., and D’Amico, S., “Adaptive End-to-End Architecture for Autonomous Spacecraft Rendezvous and Proximity Operations,” AIAA SciTech 2024 Forum, AIAA Paper 2024-1040, 2024, URL: https://slab.stanford.edu/sites/g/files/sbiybj25201/files/media/file/rpokit_scitech_2024_v3_1.pdf

[4] “Northrop Grumman Selected for In-Space Demonstration of Refueling Technologies,” Northrop Grumman Newsroom, Apr. 2, 2025, URL: https://news.northropgrumman.com/satellites/Northrop-Grumman-Selected-for-In-Space-Demonstration-of-Refueling-Technologies

# APPENDIX

## Attributions Table

Table 1: Attributions Table

| Contributor | Primary Contribution(s) | Secondary Contribution(s) |
| --- | --- | --- |
| Ryan Coder | 2.0, R1.6, R2.3, R3.3, R3.7, Org Chart | Proofread Document |
| Zachary McKissor | Itemized Budget |  |
| Charles Allen | 5.1, Nomenclature and Abbreviations, 3.0 | Proofread Document |
| Tanner Whitney | Abstract, 1.0, 5.3, Appendix | Proofread 2.0, 3.0, 4.0, 5.0 |
| Owen Glascoe | 2.0, 4.0, 5.0, 5.2, Verification Matrix | Proofread Document |
| Isaiah Gale | 3.1, 5.2 | Proofread Document |

## AI Attributions Table

Table 2: AI Attributions Table

| Contributor | AI Agent | How It Was Used |
| --- | --- | --- |
| Ryan Coder |  |  |
| Zachary McKissor |  |  |
| Charles Allen |  |  |
| Tanner Whitney |  |  |
| Owen Glascoe | ChatGPT | Rewrote 2.0 to be clearer and more concise. |
| Isaiah Gale |  |  |

## Acknowledgements

The PRISM team would like to express our sincere gratitude to Dr. Siwei Fan and Dr. Matthew Haslam for their guidance, support, and expertise throughout the course of this Capstone project. Their dedication to fostering both technical excellence and professional growth has been invaluable in shaping our work and ensuring its success. Dr. Fan’s deep knowledge of aerospace systems and Dr. Haslam’s thoughtful mentorship have challenged us to think critically, refine our designs, and approach complex problems with creativity and discipline. We are grateful for their commitment to our learning and for providing us with the opportunity to contribute meaningfully to the field of space systems engineering.

## Organizational Chart

Figure 5: PRISM's Organizational Chart

## Hours to Date

Table 3: PRISM Hours to Date

|  | Total Hours Outside of Class |
| --- | --- |
| Tanner Whitney | 6.5 |
| Ryan Coder | 6 |
| Owen Glascoe | 4.5 |
| Thomas Allen | 2.5 |
| Isaiah Gale | 3.5 |
| Zachary McKissor | 0.5 |

## Itemized Budget

Table 4: Itemized Budget

| System | Item | Allocated | Spent |
| --- | --- | --- | --- |
| MockSat | Motors (8) | $ 87.92 | $ - |
|  | Propellers (8) | $ 3.98 | $ - |
|  | ESCs (8) | $ 69.98 | $ - |
|  | PWM Board | $ 8.99 | $ - |
|  | Raspberry Pi 5 | $ 0.00 | $ - |
|  | IMU | $ 0.00 | $ - |
|  | 550mAh LiHV Battery (2) | $ 39.98 | $ - |
|  | 6200mAh LiPo Battery | $ 24.99 | $ - |
|  | DC-DC Voltage Regulator | $ 7.98 | $ - |
|  | Power Distribution Boards (2) | $ 11.98 | $ - |
|  | PLA Frame | $ 0.00 | $ - |
|  | Button Switch Modules (2) | $ 13.50 | $ - |
|  | LCD Display | $ 8.99 | $ - |
|  | Misc Wiring & Extra Components | $ 25.00 | $ - |
| Air Table | Blower | $ 195.94 | $ - |
|  | 3 ft x 3 ft x ½ in Aluminum Sheet | $ 667.72 | $ - |
|  | 2 ft x 4 ft x 16 ft Lumber (4) | $ 44.68 | $ - |
|  | 8 in x 8 in to 6 in Register Box | $ 17.98 | $ - |
|  | 6 in x 8 in Aluminum Ductwork | $ 21.97 | $ - |
|  | Paint Sealant | $ 24.98 | $ - |
|  | Caulk Sticks (4) | $ 15.92 | $ - |
|  | ¼ in x 5 in Hex Bolts | $ 14.68 | $ - |
|  | ¼ in Hex Nuts | $ 7.22 | $ - |
| GTSS | Control Box | $ 0.00 | $ - |
|  | Raspberry Pi 3 | $ 0.00 | $ - |
|  | ArduCam Module | $ 34.99 | $ - |
|  |  | Budget Total | $ 1,450.00 |
|  |  | Total Allocated | $ 1,349.37 |
|  |  | Spent | $ - |
|  |  | Remainder | $ 1,450.00 |

## Requirements Verification Matrix

Table 5: Requirements Verification Matrix

| Requirement |  |  | Design Metric(s) | Verification Method |
| --- | --- | --- | --- | --- |
| Number | Title | Value |  |  |
| R1.1 | Operational Environment | - | - | Demonstration |
| R1.2 | Translational Maneuvers | 10 mm | Steady State Error | Test |
| R1.3 | Rotational Maneuvers | 2° | Steady State Error | Test |
| R1.4 | ∆V Budget | 10 m/s | Linear Velocity | Demonstration |
| R1.5 | Dynamic Environment | 0.098 m/s2 | Coast Deceleration | Test |
| R1.6 | Repeatability | 5 % | Maneuver Time | Test |
| R2.1 | Dynamic Environment | ≤ 0.098 m/s2 | Coast Acceleration | Free Coast Motion Test |
| R2.2 | Freedom of Motion | ≥ 1.0 m2 | Table Area | Inspection |
| R2.3 | Table Flatness | ≤ ± 0.1 mm | Table Deflection | Laser Test |
| R2.3 | Payload | 0.0154 kg/cm2 | Weight to Area Ratio | Static Load Test |
| R2.4 | Air Gap | 0.18 mm ≤ x ≤ 0.4 mm | Air Gap Height | Laser Test |
| R3.1 | IMU Accuracy | 10 mm | Steady State Error | Test |
| R3.2 | Motor Actuation | 1 N | Thrust Measurement | Test |
| R3.3 | Response Time | 100 ms | IMU Response Time | Test |
| R3.4 | Rotational Motion | - | - | Demonstration |
| R3.5 | Attitude Control | ≤ 2° | MockSat Attitude | Test |
| R3.6 | Linear Acceleration | 5 m/s2 | Accelerometer Measurement | Test |
| R3.7 | Angular Acceleration | ≥ 2°/s2 | Accelerometer Measurement | Test |
| R3.8 | Thrust Operation | ≥ 2.5 min | - | Sustained Burn Demonstration |
| R4.1 | Data Logging | ≤ 0.5 s | Time Measurement | Log Speed Test |
| R4.2 | Field of View | ≥ 1 m2 | - | Demonstration |
| R4.3 | Translational Accuracy | ≤ 10 mm | Position Accuracy | Grid Test |
| R4.4 | Attitude Accuracy | ≤ 2° | Attitude Accuracy | Grid Test |